| Construction Industry | Ministry of Employment and Labor Gwangju Regional Office of Employment and Labor |
| --- | --- |

Occupational Safety, This Must Be Abided By.

Key Safety Rules for Workers

| Safety Rules to Prevent Fall Accidents. Fall Accidents: Safety rule during the work of scaffolding/work plate ○ Works and movements at the scaffolding must be conducted on top of work plates. ○ You must not step over or arbitrarily disassemble the safety guard rail installed at the scaffolding. ○ During temporary disassembly of safety guard rail, you must put on the safety harness and safety helmet. |  |
| --- | --- |
| Fall Accidents: Safety rule during the work of portable scaffold ○ You must wear the safety harness and safety helmet while on duty, and you have to refuse the portable scaffold worlds without safety guard rails. ○ You must not step over or disassemble the safety guard rail of the portable scaffold. ○ When the portable scaffold is moved, you must step down to the ground from the work plate. Fall Accidents: Safety rule during the work of hanging scaffold ○ A rope of hanging scaffold needs to be tightly fastened to the dedicated hook. ○ You need to install a separate rescue rope and attach the safety harness to the rescue rope. ○ You must refuse the hanging scaffolding works without a safety harness. |  |

Fall Accidents: Safety rule during the work at ladders

○ Portable ladders must not be used unless work plates and fall-prevention nets cannot be installed.

○ When going upward/downward by using a ladder, you need to take measures to prevent the ladder from falling or slipping.

* Fall-prevention: Installation of outrigger or fastening via connection to the facility; work by a group of 2 workers

* Slip-prevention: Attachment of anti-slipping pads, etc.

○ When using a ladder, you need to put on a safety harness and safety helmet.

| Fall/Collapse Accidents: Safety rule during the work of form/timbering ○ Form timbering must be assembled according to the assembly drawing after its structure is reviewed. ○ When form timbering is assembled/disassembled, fall-prevention measures must be verified. ○ If deformation displacement is verified during concrete cast work, you must evacuate immediately. Fall Accidents: Safety rule during the work of steel frame ○ When working on top of steel frames, you must wear and fasten the safety harness. ○ 2 tail-pieces needs to be used when steel frames are lifted. ○ You must not access the space beneath the steel frames which are being assembled or lifted. |  |
| --- | --- |
| Fall/Trapping Accidents: Safety rule during the work of aerial work platform ○ You must put on safety helmet/harness at the work platform. ○ You must not break away from the work platform. ○ Safety device preventing excessive rise must not be arbitrarily disassembled or operated. |  |

Safety Rules to Prevent Collision/Striking

| Collision: Safety rule during the work of excavator ○ You must not enter within the operating radius of excavators. ○ A person operating the excavator must fasten a seat belt. ○ Safety pins must be fastened when device, i.e. bucket, breaker and clam shell, is attached or replaced. |  |
| --- | --- |
| Collision: Safety rule during the work of truck ○ Do not access the place prone for risks of contacts with trucks. ○ You must not transit through the route of vehicles, and be sure to follow the signals given by a vehicle guardian. ○ When unavoidably climbing at the cargo box, be cautious of fall accidents by fastening a safety harness. |  |

Striking: Safety rule during the work of mobile crane

○ Do not use worn-out or deformed ropes for a hanger rope.

○ Outriggers of a crane need to be installed at a location without risks of soil settlement.

* Location with risks of soil settlement: Earth and sand ground that is not hard, sidewalk block, drip box, etc.

○ You must not access beneath the cargo that is being lifted.

Safety Rules to Prevent Collapse

Collapse: Safety rule during the work at excavating slope

○ You must comply with operation methods and orders according to directions given by a supervisor.

○ Stay cautious not to contact with excavation machinery by following the signals given by a guardian.

○ When signs of earth and sand collapse, i.e. ground rupture and groundwater rise, are detected, you must stop operations and evacuate.

Fall/Collapse Accidents: Safety rule during the work of sheathing timbering

○ Be sure to check and comply with the installation methods and orders specified in the assembly drawings.

○ You must wear a safety harness when working at a place prone for fall accidents, i.e. upper part of deck plates.

○ You must check 2 tail-pieces, possible worn-out and damages to ropes and hook release device when lifting materials.

Collapse: Safety rule during the work of tower crane

○ You must comply with operation methods and orders according to directions given by a supervisor.

○ You must check 2 tail-pieces, possible worn-out and damages to ropes and hook release device when lifting materials.

○ Do not access the dangerous area beneath the cargo being lifted.

Collapse: Safety rule during the work of pile driver/pile extractor

○ An unauthorized person must not approach the site of installation and demolition.

○ You must fully understand the operation methods and procedures, and follow the directions given by a supervisor.

○ Be sure to put on a safety harness when going up to the upper part of leader to check a pile driver.

| Collapse: Safety rule during the work of lift for construction ○ An access door of the lift may not be opened arbitrarily except for entry/exit. ○ No load in excess of maximum live load is allowed for delivery vehicles. ○ Do not arbitrarily release the safety device of the lift. |  |
| --- | --- |