| Manufacturing/Others | Ministry of Employment and Labor Gwangju Regional Office of Employment and Labor |
| --- | --- |

Occupational Safety, This Must Be Abided By.

Key Safety Rules for Workers

| Fork lift | √ Driving by a person with qualification or license √ Compliance with removal of vehicle key when not in operation √ Placement of guardians and compliance with signals √ Wearing of seat safety belt and compliance with speed limit |  |
| --- | --- | --- |
| ② Aerial work platform | √ Wearing of safety helmet/safety harness and fastening of safety harness hook √ Movement after bumps on the route are checked √ Prohibition of acts to pull or push when riding an aerial work platform |  |
| ③ Press | √ Check the operation of optoelectric protective device. √ Prohibition of nullification of two-hand control safety device √ Use of safety block during attachment of press die |  |
| ④ Injection molding machine | √ Verification of interlink with gate guard √ Verification of location and operation of emergency stop device √ Power shut-off and attachment of 'Do Not Operate' sign during metal mold replacement work or repair/cleaning |  |
| ⑤ Heavy material handling | √ Prohibition of access by unauthorized personnel √ Procurement of sufficient safety distance between workers √ Inspection and replacement of lifting wire rope |  |
| ⑥ Industrial robot | √ Prohibition of access within operating range of robots √ Verification of operation of protective device √ Operation must be suspended during inspection/repair works. |  |

Occupational Safety, This Must Be Abided By.

Key Safety Rules for Workers

| ⑦ Welding work | √ Wearing of face shields √ Re-check the gas during the gas engagement. √ Verification of nearby combustible materials, installation of preventive cloth for spark scattering and confirmation of location of fire extinguisher |  |
| --- | --- | --- |
| ⑧ Electric work | √ Wearing of insulated protective equipment √ Verification of connection with ground cable √ Verification of installation/operation of circuit breaker |  |
| ⑨ Handling of harmful substances requiring management | √ Understanding of material safety data sheets √ Wearing of proper protective equipment √ Wearing of safety goggles in cases of risks for scattering of materials |  |
| ⑩ Confined space | √ Wearing of protective equipment, i.e. air respirator or air-supplied mask √ Access after measurement of oxygen/harmful gas concentration and ventilation √ Prohibition of isolated works and availability of guardian and contact system |  |
| ⑪ Logging work | √ Prohibition of works beneath hanging logs √ Comply with the notch angle and depth during the operation by deciding the wood type and felling-down direction √ Removal of nearby obstacles and procurement of safe evacuation routes and shelters |  |
| ⑫ Maintenance work | √ Power shut-off prior to operation √ Attachment of 'Do Not Operate' sign √ Check the surroundings and operate following the work |  |